BULK

Pink Batts Wall Insulation

Bulk insulation product

# 1. Product description

Pink® Batts is a thermal and acoustic insulation made from flexible and resilient glasswool for residential wall applications. It is designed to fit standard timber and steel studs, helping to improve energy efficiency.

## Key features

Firm to fit Friction fit to stay in place without slumping.

Recover to their natural thickness quickly.

Australian made using up to 80% recycled content.

Codemark certified Assurance that product meets requirements of National Construction Code (NCC).

# 2. Applications and selection

wall

## Selection checklist

Confirm the application matches the family.

Confirm the target rating and construction build-up.

Confirm available depth against product dimensions.

Check NCC, fire, BAL or acoustic requirements with a qualified reviewer.

Record the suburb/postcode for climate-zone checks.

# 3. Current catalogue range

| SKU | Product | Published rating |
| --- | --- | --- |
| FI PW Batt 168 | Pink Batts Wall Insulation | R1.5 |
| FI PB 20430 | PinkBatt R2.0 1160*430*90 | R2.2, R2.0 |
| FI PB 20580 | PinkBatt R2.0 1160*580*90 | R2.17, R2.0 |
| FI PB 20430HD | PinkBatt R2.0HD 1160*430*70 | R2.2, R2.0 |
| FI PB 20580HD | PinkBatt R2.0HD 1160*580*70 | R2.17, R2.0 |
| FI PB 25430HD | PinkBatt R2.5HD 1160*430*90 | R2.7, R2.5 |
| FI PB 25580HD | PinkBatt R2.5HD 1160*580*90 | R2.67, R2.5 |
| FI PB 40430HD | PinkBatt R4.0HD 1160*430*140 | R4.5, R4.0 |
| FI PB 40580HD | PinkBatt R4.0HD 1160*580*140 | R4.17, R4.0 |
| FL-4006277 | PINK BATTS WALL R1.5 1160X430X70 24PK | R1.5 |
| FL-4006278 | PINK BATTS WALL R1.5 1160X580X70 24PK | R1.5 |

# 4. Technical data

| Property | Value | Standard |
| --- | --- | --- |
| R-value | R1.5 | m2K/W |
| R-value | R2.0 | m2K/W |
| R-value | R2.0 HD | m2K/W |
| R-value | R2.5 HD | m2K/W |
| R-value | R4.0 HD | m2K/W |
| Maximum service temperature | 340 °C | ASTM C411/C447 |
| Thermal resistance R-value | Complies | AS/NZS 4859.1 |
| Moisture absorption | < 0.2% | % by volume |

Extracted from manufacturer datasheet: https://insulation.com.au/wp-content/uploads/2024/TDS-Pink-Batts-Wall-Rev12-010225.pdf

# 5. Fire, testing and compliance context

Combustibility AS1530.1 Non-combustible; Early Fire Hazard Indices: Ignitability Index 0, Spread of Flame Index 0, Heat Evolved Index 0, Smoke Developed Index 0–1; BAL Compliance AS3959 Low–FZ

NCC / project compliance: conditional - project-specific evidence required. BAL: not verified.

# 6. Installation overview

Product must be kept dry and not be exposed to weather in any condition including prior, during and after installation..

Product must be correctly installed in the right stud width and depth..

Product must be installed without compression to keep its claimed R-value..

If used with any other products except Sisalation, confirmation of suitability must be reviewed..

Follow the Installation Guidelines available from insulation.com.au.

# 7. Safety, handling and SDS status

SDS: to be sourced. Confirm the current SDS before handling or cutting.

# 8. Sustainability and indoor environment

Australian made using up to 80% recycled content; Zero ODP insulation; no harmful levels of Volatile Organic Compounds (VOCs) released.

# 9. Warranty, returns and support

No product-specific warranty term is asserted in this draft. Refer to the manufacturer's general terms.

# 10. Specification starting point

Insulation shall be Pink Batts Wall Insulation (bulk) by Fletcher, selected to suit the confirmed application and required rating. Exact product, thickness and compliance to be confirmed by the project reviewer against the current manufacturer TDS before ordering.

# 11. Public document register

SRC-01 Manufacturer datasheet: https://insulation.com.au/product/pink-batts-insulation/.

SRC-02 Safety data sheet: to be sourced.

# 12. Technical-review actions before publication

Confirm the exact product TDS deep link and re-validate all technical values.

Confirm SDS currency and handling guidance.

Approve environmental and warranty wording with the manufacturer.

Human review of NCC, fire, BAL and acoustic claims remains mandatory.

Draft generated 2026-09-05 from the internal knowledge base. Technical values and claims remain subject to manufacturer review before publication.