# Acoustic walls

## GENERAL100

NOTE :	THE NATIONAL OCCUPATIONAL HEALTH AND SAFETY COMMISSION PRODUCED IN AUGUST 1988 A CODE OF PRACTICE AND GUIDANCE NOTES ON ASBESTOS.  IN THE PREFACE TO THAT DOCUMENT, PART 3, Clause 3.1, second paragraph states :

“ASBESTOS REMOVAL MAY NOT BE IMMEDIATELY NECESSARY, BUT MUST BE COMPLETED BEFORE A STRUCTURE OR PART OF A STRUCTURE IS DEMOLISHED.”

### General

A.	Scope

The work of this section includes but is not limited to the following items:

QuietWave acoustic plasterboard is the most economic 6-star acoustic wall system available. The thinnest and lightest way to achieve the highest acoustic performance, QuietWave is a class above minimum NCC requirements.

Effective for low frequencies to counter everyday noise from home audio and theatre systems, QuietWave consists of a flexible visco-elastic membrane and patented constrained layer membrane, combined with vibration damping material. Easily penetrated without compromising performance, a finished QuietWave wall is solid and has similar resonance to masonry.

At just 1.2 mm thick QuietWave provides a saving of 50% in the width of the wall against a comparable 6-star wall system and can be easily installed by just one person.

Supply and installation of QuietWave wall system – SINGLE STUD CONSTRUCTION

Timber Stud sections

| Stud mm + plasterboard mm | Wall mm | Acoustic opinion Rw+Ctr (Decibels) | FRL |
| --- | --- | --- | --- |
| 70-10 SD | 113 | 54 db | - |
| 70-13 SD | 125 | 51 db | - |
| 70-13 FC | 125 | 53 db | -/120/120 |
| 70-16 FC | 137 | 55 db | -/120/120 |
| 70-10 SC | 113 | 52 db | - |
| 70-13 SC | 125 | 56 db | - |
| 75-10 SD | 118 | 51 db | - |
| 75-13 SD | 130 | 54 db | - |
| 75-13 FC | 130 | 56 db | -/120/120 |
| 75-13 FC | 142 | 52 db | -/120/120 |
| 75-10 SC | 118 | 56 db | - |
| 75-13 SC | 130 |  |  |
| 90-10 SD | 133 | 49 db | - |
| 90-13 SD | 145 | 53 db | - |
| 90-13 FC | 145 | 55 db | -/120/120 |
| 90-16 FC | 157 | 57 db | -/120/120 |
| 90-10 SC | 133 | 54 db | - |
| 90-13 SC | 145 | 58 db | - |
| 120-10 SD | 163 | 52 db | - |
| 120-13 SD | 175 | 55 db | - |
| 120-13 FC | 175 | 57 db | -/120/120 |
| 120-16 FC | 187 | 59 db | -/120/120 |
| 120-10 SC | 163 | 56 db | - |
| 120-13 SC | 175 | 60 db | - |

Fire rated > 50 RW + ctr walls

Standard >50 RW + ctr walls

Legend:

SD: Standard plasterboard

FC: Fyrchek plasterboard

SC: Soundchek plasterboard

Note:

Gap between ceiling and wall to be filled with Bostik Fireban 1 or equal in fire rated wall

Steel Stud sections

| Stud mm + plasterboard mm | Wall mm | Acoustic opinion Rw+Ctr (Decibels) | FRL |
| --- | --- | --- | --- |
| 51-10 SD | 94 | 45 db | - |
| 51-13 SD | 106 | 48 db | - |
| 51-13 FC | 106 | 51 db | -/120/120 |
| 51-16 FC | 118 | 53 db | -/120/120 |
| 51-10 SC | 94 | 49 db | - |
| 51-13 SC | 106 | 53 db | - |
| 64-10 SD | 107 | 47 db | - |
| 64-13 SD | 119 | 50 db | - |
| 64-13 FC | 119 | 53 db | -/120/120 |
| 64-13 FC | 131 | 55 db | -/120/120 |
| 64-10 SC | 107 | 51 db | - |
| 64-13 SC | 119 | 55 db | - |
| 76-10 SD | 119 | 48 db | - |
| 76-13 SD | 131 | 51 db | - |
| 76-13 FC | 131 | 54 db | -/120/120 |
| 76-16 FC | 143 | 56 db | -/120/120 |
| 76-10 SC | 119 | 52 db | - |
| 76-13 SC | 131 | 56 db | - |
| 92-10 SD | 135 | 49 db | - |
| 92-13 SD | 147 | 53 db | - |
| 92-13 FC | 147 | 55 db | -/120/120 |
| 92-16 FC | 159 | 57 db | -/120/120 |
| 92-10 SC | 135 | 54 db | - |
| 92-13 SC | 147 | 68 db | - |

Fire rated > 50 RW + ctr walls

Standard >50 RW + ctr walls

Legend:

SD: Standard plasterboard

FC: Fyrchek plasterboard

SC: Soundchek plasterboard

Note:

Gap between ceiling and wall to be filled with Bostik Fireban 1 or equal in fire rated wall

Supply and installation of QuietWave wall system – DISCONTINUOUS CONSTRUCTION

Timber Stud sections

| Stud mm + plasterboard mm | Wall mm | Acoustic opinion Rw+Ctr (Decibels) | FRL |
| --- | --- | --- | --- |
| 70-10 SD | 208 | 54 db | - |
| 70-13 SD | 220 | 57 db | - |
| 70-13 FC | 220 | 59 db | -/120/120 |
| 70-16 FC | 232 | 61 db | -/120/120 |
| 70-10 SC | 208 | 58 db | - |
| 70-13 SC | 220 | 62 db | - |
| 75-10 SD | 213 | 54 db | - |
| 75-13 SD | 225 | 57 db | - |
| 75-13 FC | 225 | 59 db | -/120/120 |
| 75-13 FC | 237 | 61 db | -/120/120 |
| 75-10 SC | 213 | 58 db | - |
| 75-13 SC | 225 | 62 db |  |
| 90-10 SD | 243 | 55 db | - |
| 90-13 SD | 255 | 58 db | - |
| 90-13 FC | 255 | 60 db | -/120/120 |
| 90-16 FC | 267 | 62 db | -/120/120 |
| 90-10 SC | 243 | 59 db | - |
| 90-13 SC | 255 | 63 db | - |
| 120-10 SD | 303 | 57 db | - |
| 120-13 SD | 315 | 60 db | - |
| 120-13 FC | 315 | 63 db | -/120/120 |
| 120-16 FC | 327 | 65 db | -/120/120 |
| 120-10 SC | 303 | 61 db | - |
| 120-13 SC | 315 | 65 db | - |

Fire rated > 50 RW + ctr walls

Standard >50 RW + ctr walls

Legend:

SD: Standard plasterboard

FC: Fyrchek plasterboard

SC: Soundchek plasterboard

Note:

Gap between ceiling and wall to be filled with Bostik Fireban 1 or equal in fire rated wall

Steel Stud sections

| Stud mm + plasterboard mm | Wall mm | Acoustic opinion Rw+Ctr (Decibels) | FRL |
| --- | --- | --- | --- |
| 51-10 SD | 165 | 52 db | - |
| 51-13 SD | 177 | 55 db | - |
| 51-13 FC | 177 | 57 db | -/120/120 |
| 51-16 FC | 189 | 59 db | -/120/120 |
| 51-10 SC | 165 | 56 db | - |
| 51-13 SC | 177 | 59 db | - |
| 64-10 SD | 191 | 53 db | - |
| 64-13 SD | 203 | 56 db | - |
| 64-13 FC | 203 | 59 db | -/120/120 |
| 64-13 FC | 215 | 61 db | -/120/120 |
| 64-10 SC | 191 | 57 db | - |
| 64-13 SC | 203 | 61 db | - |
| 76-10 SD | 215 | 54 db | - |
| 76-13 SD | 227 | 57 db | - |
| 76-13 FC | 227 | 60 db | -/120/120 |
| 76-16 FC | 239 | 62 db | -/120/120 |
| 76-10 SC | 215 | 58 db | - |
| 76-13 SC | 227 | 62 db | - |
| 92-10 SD | 247 | 55 db | - |
| 92-13 SD | 259 | 58 db | - |
| 92-13 FC | 259 | 60 db | -/120/120 |
| 92-16 FC | 271 | 63 db | -/120/120 |
| 92-10 SC | 247 | 60 db | - |
| 92-13 SC | 259 | 63 db | - |

Fire rated > 50 RW + ctr walls

Standard >50 RW + ctr walls

Legend:

SD: Standard plasterboard

FC: Fyrchek plasterboard

SC: Soundchek plasterboard

Note:

Gap between ceiling and wall to be filled with Bostik Fireban 1 or equal in fire rated wall

Supply and installation of QuietWave wall system – SINGLE STAGGERED STUD CONSTRUCTION

Timber Stud sections

| Stud mm + plasterboard mm | Wall mm | Acoustic opinion Rw+Ctr (Decibels) | FRL |
| --- | --- | --- | --- |
| 90-10 SD | 133 | 50 db | - |
| 90-13 SD | 145 | 53 db | - |
| 90-13 FC | 145 | 55 db | -/120/120 |
| 90-16 FC | 157 | 57 db | -/120/120 |
| 90-10 SC | 133 | 54 db | - |
| 90-13 SC | 145 | 58 db | - |
| 120-10 SD | 163 | 52 db | - |
| 120-13 SD | 175 | 55 db | - |
| 120-13 FC | 175 | 57 db | -/120/120 |
| 120-16 FC | 187 | 59 db | -/120/120 |
| 120-10 SC | 163 | 56 db | - |
| 120-13 SC | 145 | 60 db | - |

Fire rated > 50 RW + ctr walls

Standard >50 RW + ctr walls

Legend:

SD: Standard plasterboard

FC: Fyrchek plasterboard

SC: Soundchek plasterboard

Note:

Gap between ceiling and wall to be filled with Bostik Fireban 1 or equal in fire rated wall

B.	Examine documents:  Examine parts of the drawings and this specification for requirements which affect the work of this specification.  In particular, take note of other trades involved.

C.	Co-ordination:  Co-ordinate with other trades affecting or affected by work of this section, co-operating as necessary to ensure steady and satisfactory progress of the work.

### Other trades

Co-ordinate and co-operate with the following trades:

-Disconnection of existing services by appropriate other trades if necessary.

-Carpentry

-Framing work

-Suspended ceiling system

-Plumbing

-Electrical alterations and installations

-Mechanical ventilation alterations and installation

-Fire protection

-Data & communication

### Quality Assurance

Installation should be executed as per specification by experienced installers.

### Compliances

-Comply with applicable portions of the Australian Standards

-Comply with applicable clauses of the current version of the NCC.

-Follow instructions contain in Engineers reports and Engineers specifications

-Comply with Applicable building regulations and any statutory authority having jurisdiction,

-Comply with Local council codes, regulations and approval conditions

-Comply with site specific W&S procedure

### Public and Property Protection

Provide measures required by municipal and state ordinances, laws, and regulations for the protection of surrounding property, footpaths, streets, kerbs, the public, occupants and workmen during installation operations.

Comply with the above ordinances, laws etc. in carrying out measures including barricades, fences, warning lights and signs, rubbish chutes, etc..

Make good to original condition, damage to substrate which results from installation operations. Perform restoration work without expense to the Proprietor.

### Services

Before installation commences ensure that all related services work have been completed. If some services need to be altered during the course of the installation, ensure that all services trade-person have been notified and that required coordination work has been organised.

### Photographic Record

Arrange for photographic record of the progress of installation. Produce for the Proprietor if requested JPEG files of “before and after” installation of the work involved.

## Products200

### QuietWave acoustic membrane

QuietWave Acoustic Membrane from Acoustica Pty Ltd

Rolls: 1300 x 5400 x 1.2mm

Visit site and inspect conditions, comparing conditions to the Drawings before delivery of materials to site.  Report any discrepancy or unsuitability of substrate.

Start of work means total acceptance of conditions.

All installation drawings outlined in above scope are to be used as a guide only. For specific applications and instructions, please contact Acoustica below.

Contact:

25 Plasser Crescent

North St Marys, NSW, 2760

Tel: 02 95502900

Email:

info@acoustica.com.au

### Equipment

A.	Supply equipment required to perform the work of sufficient capacity to meet the stated completion date.

B.	Provide access equipment necessary to execute the work

C.	Provide disposal containers for disposal required.

D.	No containers may be located on public streets or pavements without obtaining required municipal permits for same.  Co-operate with Sub-Contractors doing work in or near container locations to prevent disruption of their work.

## EXECUTION300

### Examination

Visit site and inspect conditions, comparing conditions to the Drawings before delivery of materials to site.  Report any discrepancy or unsuitability of substrate.

Start of work means total acceptance of conditions.

### Preparation

Co-ordinate with and ensure preparatory work by other trades is done prior to commencement of work.

### Installation

QuietWave membrane    to    be    sandwiched    between    2    layers of plasterboard, MDF, yellow tongue, plywood or other lining material)

#### Install stud and tracks as per manufacturer’s instructions.

Verify that all framing work has been installed in accordance with the drawings.

Install acoustic insulation as per specified on the drawings...see previous

#### Layer 1:  Lining

-Install 1st substrate layer (Plasterboard, Ply, or other Lining) as per   manufacturer’s instructions.

-lining must be cut accurately leaving only 2mm gaps around walls or ceiling edges

-Seal all joints and edges with silicone

#### Layer 2:  QuietWave membrane

-Install 1st substrate layer (Plasterboard, Ply, or other Lining) as per   manufacturer’s instructions.

-lining must be cut accurately leaving only 2mm gaps around walls or ceiling edges

-Seal all joints and edges with silicone

#### Similar logic for ceiling installation

#### Second layer of plasterboard

4.1          	Put up the second layer of plasterboard in a vertical direction as per

manufacturer’s    instructions    or    ensure    that    the    first    layer    of plasterboard joints

do not coincide with the second layer joints.

4.2          	Seal the    joints.

4.3          	Cut the    edges    neatly    leaving    a    maximum 2mm gap around the wall.

4.4  	Seal  edges    with resilient    sealant    (Silicone    or    No More    Gaps)

### Clean up as the work proceeds.

### Installation Particulars

### Finishing Details

Install as per detail drawings

### Protection

Protect finished work until work has been. Make good damage in every respect at no additional cost to the Proprietor.

### Cleaning

Clean exposed surfaces.  Remove all debris and  waste and dispose

### Completion

Complete contracted work in accordance with contract documents and written variation orders issued by the Architect or Superintendent.

END OF SECTION